THE IMAGINATION UNIVERSITY PROGRAMME

RVfpga Lab 2

RISC-V Assembly Language

# INTRODUCTION

Programming in higher-level languages such as C, Java, and Python are efficient for the programmer. These higher-level languages are translated into assembly language, which is a group of simple instructions. Sometimes performance- or timing-critical sections of code are written in assembly to guarantee specific timing or reduce computation time. This lab shows you how to create a RISC-V assembly language program that you can run on the RVfpga System using PlatformIO. We first give a brief overview of RISC-V assembly and then show how to create and run an assembly program on RVfpgaNexys (remember that you can also execute the programs on simulation by using Verilator or Whisper). Then we provide exercises for you to practice writing your own RISC-V assembly programs.

# RISC-V Assembly Language Overview

RISC-V assembly language includes simple instructions that are used to implement higher-level code. For example, some common RISC-V instructions include the add, sub, and mul instructions that add, subtract or multiply two operands.

The basic types of RISC-V instructions are: computational (arithmetic, logical, and shift) instructions, memory operations, and branches/jumps. The most common RISC-V instructions are given in Table 1. Instructions use operands that are located in registers or memory or that are encoded as a constant (i.e., immediate). RISC-V includes 32 32-bit registers. Table 2 lists the names of the 32 RISC-V registers. They can be specified by either their name (for example, zero, s0, t5, etc.) or their register number (i.e., x0, x8, x30). Programmers typically use register names which retains some information about the typical purpose of the register. For example, the saved registers, s0-s11, are typically used for program variables, while the temporary registers, t0-t6 are used for temporary calculations. The zero register (x0) always contains the value 0, as this is a value commonly needed in programs. The other registers have specific uses as well, as shown in Table 2, but in this lab, you need only use the zero register and the temporary and saved registers.

Table 1. Common RISC-V assembly instructions

|  |  | RISC-V Assembly |  | Description |  | Operation |
| --- | --- | --- | --- | --- | --- | --- |
| Computational | add s0, s1, s2 |  | Add |  | s0 = s1 + s2 |  |
|  | sub s0, s1, s2 |  | Subtract |  | s0 = s1 - s2 |  |
|  | addi t3, t1, -10 |  | Add immediate |  | t3 = t1 – 10 |  |
|  | mul t0, t2, t3 |  | 32-bit multiply |  | t0 = t2 * t3 |  |
|  | div s9, t5, t6 |  | Division |  | t9 = t5 / t6 |  |
|  | rem s4, s1, s2 |  | Remainder |  | s4 = s1 % s2 |  |
|  | and t0, t1, t2 |  | Bit-wise AND |  | t0 = t1 & t2 |  |
|  | or t0, t1, t5 |  | Bit-wise OR |  | t0 = t1 ¦ t5 |  |
|  | xor s3, s4, s5 |  | Bit-wise XOR |  | s3 = s4 ^ s5 |  |
|  | andi t1, t2, 0xFFB |  | Bit-wise AND immediate |  | t1 = t2 & 0xFFFFFFFB |  |
|  | ori t0, t1, 0x2C |  | Bit-wise OR immediate |  | t0 = t1 ¦ 0x2C |  |
|  | xori s3, s4, 0xABC |  | Bit-wise XOR immediate |  | s3 = s4 ^ 0xFFFFFABC |  |
|  | sll t0, t1, t2 |  | Shift left logical |  | t0 = t1 << t2 |  |
|  | srl t0, t1, t5 |  | Shift right logical |  | t0 = t1 >> t5 |  |
|  | sra s3, s4, s5 |  | Shift right arithmetic |  | s3 = s4 >>> s5 |  |
|  | slli t1, t2, 30 |  | Shift left logical immediate |  | t1 = t2 << 30 |  |
|  | srli t0, t1, 5 |  | Shift right logical immediate |  | t0 = t1 >> 5 |  |
|  | srai s3, s4, 31 |  | Shift right arithmetic immediate |  | s3 = s4 >>> 31 |  |
| Memory | lw s7, 0x2C(t1) |  | Load word |  | s7 = memory[t1+0x2C] |  |
|  | lh s5, 0x5A(s3) |  | Load half-word |  | s5 = SignExt(memory[s3+0x5A]15:0) |  |
|  | lb s1, -3(t4) |  | Load byte |  | s1 = SignExt(memory[t4-3]7:0) |  |
|  | sw t2, 0x7C(t1) |  | Store word |  | memory[t1+0x7C] = t2 |  |
|  | sh t3, 22(s3) |  | Store half-word |  | memory[s3+22]15:0 = t315:0 |  |
|  | sb t4, 5(s4) |  | Store byte |  | memory[s4+5]7:0 = t47:0 |  |
| Branch | beq s1, s2, L1 |  | Branch if equal |  | if (s1==s2), PC = L1 |  |
|  | bne t3, t4, Loop |  | Branch if not equal |  | if (s1!=s2), PC = Loop |  |
|  | blt t4, t5, L3 |  | Branch if less than |  | if (t4 < t5), PC = L3 |  |
|  | bge s8, s9, Done |  | Branch if greater than or equal |  | if (s8>=s9), PC = Done |  |
| Pseudoinstructions | li s1, 0xABCDEF12 |  | Load immediate |  | s1 = 0xABCDEF12 |  |
|  | la s1, A |  | Load address |  | s1 = Memory address where variable A is stored |  |
|  | nop |  | Nop |  | no operation |  |
|  | mv s3, s7 |  | Move |  | s3 = s7 |  |
|  | not t1, t2 |  | Not (Invert) |  | t1 = ~t2 |  |
|  | neg s1, s3 |  | Negate |  | s1 = -s3 |  |
|  | j Label |  | Jump |  | PC = Label |  |
|  | jal L7 |  | Jump and link |  | PC = L7; ra = PC + 4 |  |
|  | jr s1 |  | Jump register |  | PC = s1 |  |

In addition to actual RISC-V instructions, RISC-V includes pseudoinstructions (as shown in the bottom of Table 1), instructions that are not really RISC-V instructions but that are commonly used by programmers. Pseudoinstructions are implemented using one or more real RISC-V instruction. For example, the move pseudoinstruction (mv s1, s2) copies the contents of s2 and puts it in s1. It is implemented using the real RISC-V instruction: addi s1, s2, 0.

Table 2. RISC-V registers

| Name | Register Number | Use |
| --- | --- | --- |
| zero | x0 | Constant value 0 |
| ra | x1 | Return address |
| sp | x2 | Stack pointer |
| gp | x3 | Global pointer |
| tp | x4 | Thread pointer |
| t0-2 | x5-7 | Temporary variables |
| s0/fp | x8 | Saved register / Frame pointer |
| s1 | x9 | Saved register |
| a0-1 | x10-11 | Function arguments / Return values |
| a2-7 | x12-17 | Function arguments |
| s2-11 | x18-27 | Saved registers |
| t3-6 | x28-31 | Temporary variables |

# The commands that start with a period are assembler directives. They are commands to the assembler rather than code to be translated by it. They tell the assembler where to place code and data, specify text and data constants for use in the program, and so forth. Table 3 shows the main assembler directives of RISC-V (The RISC-V Reader: An Open Architecture Atlas, Patterson & Waterman, © 2017).

Table 3. RISC-V main directives

| Directive | Description |
| --- | --- |
| .text | Subsequent items are stored in the text section (machine code). |
| .data | Subsequent items are stored in the data section (global variables). |
| .bss | Subsequent items are stored in the bss section (global variables initialized to 0). |
| .section .foo | Subsequent items are stored in the section named .foo. |
| .align n | Align the next datum on a 2n-byte boundary. For example, .align 2 aligns the next value on a word boundary. |
| .balign n | Align the next datum on an n-byte boundary. For example, .balign 4 aligns the next value on a word boundary. |
| .globl sym | Declare that label sym is global and may be referenced from other files |
| .string “str” | Store the string str in memory and null-terminate it. |
| .word w1,…,wn | Store the n 32-bit quantities in successive memory words. |
| .byte b1,…,bn | Store the n 8-bit quantities in successive bytes of memory. |
| .space | Reserve memory space to store variables without an initial value. It is commonly used to declare the output variables, when they are not also serving as input variables. The space we want to reserve must always be expressed as a number of bytes. For example, the directive RES: .space 4 reserves four bytes (i.e. one word) that are not initialized. |
| .equ name,constant | Define symbol name with value constant. For example, .equ N,12, defines symbol N with the value 12. |
| .end | The assembler will conclude its work when it reaches the directive .end. Any text located after this directive will be ignored. |

# The examples below (see Table 4 - Table 5) show how to code some common high-level constructs in RISC-V assembly. Notice that branch instructions (beq, bne, blt, and bge) conditionally jump to a label; whereas the jump instruction (j) unconditionally jumps to a label. Single-line comments are indicated by // in C and # in RISC-V assembly.

In the first example (implementing an if/else statement, see Table 4), notice that the C code and RISC-V assembly code check for the opposite cases: the C code checks for less than (<) and the assembly equivalent checks for greater than or equal (>=).

Table 4. RISC-V Assembly Example 1: if/else statement

| // C Code int a, b, c; if (a < b) c = 5; else c = a + b; | # RISC-V Assembly # s0 = a, s1 = b, s2 = c bge s0, s1, L1 # if (a >= b) goto L1 addi s2, zero, 5 # c = 5 j L2 # jump over else block L1: add s2, s0, s1 # c = a + b L2: |
| --- | --- |

In the second example (manipulating an array of integers, see Table 5), the RISC-V assembly code uses temporary registers (t0-t3) to hold temporary values, such as the constant 100 and the base address of the data array. After initializing the registers in the first three instructions, the RISC-V assembly code checks for i >= 100 using the bge (branch if greater than or equal to) instruction; again, this is the opposite case from the C code. If that condition is met, the for loop is done. If the branch is not taken, i is less than 100 and the remaining code is executed. Notice that the index i is multiplied by 4 (using the slli t2, s0, 2 instruction) before it is added to the base address because integers (32-bit two’s complement numbers) occupy 4 bytes of memory. In RISC-V, memory is byte-addressable (i.e., each byte has its own address). If the array had been an array of characters (i.e., char data[100];), then each array element would only occupy a byte and i could be added directly to the base address to form the address of array index i, i.e., array[i].  After the array element is read, decremented by ten, and written (via the lw, addi, and sw instructions, respectively), the array index i (i.e., s0) is incremented and the program jumps back to the beginning of the for loop (using the j L5 instruction).

Table 5. RISC-V Assembly Example 2: manipulating an array of integers

| // C Code int i; int data[100]; for (i=0; i<100; i++) array[i] = array[i]-10; | # RISC-V Assembly # s0 = i, t1 = base address of data (assumed # to be at 0x300) addi s0, zero, 0 # i = 0 addi t0, zero, 100 # t0 = 100 li t1, 0x300 # base address of array L5: bge s0, t0, L7 # if (i>=100) exit loop slli t2, s0, 2 # t2 = i*4 add t2, t1, t2 # address of data[i] lw t3, 0(t2) # t3 = array[i] addi t3, t3, -10 # t3 = array[i]-10 sw t3, 0(t2) # array[i] = array[i]-10 addi s0, s0, 1 # i++ j L5 # loop L7: |
| --- | --- |

For more details about the RISC-V assembly language, refer to the RISC-V Instruction Set Manual (available here: https://github.com/riscv/riscv-isa-manual/releases/download/Ratified-IMAFDQC/riscv-spec-20191213.pdf) or a textbook such as Digital Design and Computer Architecture: RISC-V Edition, Harris & Harris, © Morgan Kaufmann 2021 or The RISC-V Reader: An Open Architecture Atlas, Patterson & Waterman, © 2017.

# Writing a RISC-V Assembly Program for RVfpga

Now you are ready to explore and practice writing RISC-V assembly programs on your own. Before you write your own programs, follow these steps to setup a PlatformIO project and create and run an assembly program on RVfpgaNexys (remember that you can also run these programs on simulation, using Verilator or Whisper):

- Create an RVfpga project

- Write a RISC-V assembly language program

- Download RVfpgaNexys onto Nexys A7 FPGA board

- Compile, download, and run assembly program

Step 1. Create an RVfpga project

Follow Step 1 from RVfpga Lab 1 – repeated here for convenience. Open VSCode by clicking on the Start button and typing VSCode and then clicking on Virtual Studio Code (see Figure 1).

Figure 1. Open VSCode

If PlatformIO does not automatically open when you start VSCode, click on the PlatformIO icon in the left menu ribbon and then click on PIO Home → Open (see Figure 2).

Figure 2. Open PlatformIO and create new project

Now in the PIO Home welcome window, click on New Project (see Figure 2).

As shown in Figure 3, name the project Project1 and choose the Board as RVfpga: Digilent Nexys A7 (start typing in RVfpga and the board will come up). Leave the default framework as WD-framework (Western Digital framework – which includes the Freedom-E SDK gcc and gdb). Unclick the Use default location and place your program in:

[RVfpgaPath]/RVfpga/Labs/Lab02

Figure 3. Name project and select board and project folder

Then click Finish at the bottom of the window (see Figure 4).

Figure 4. Finish creating project

In the Explorer pane on the left, under PROJECT1 (which you may need to expand), double-click on platformio.ini to open it (see Figure 5). This is the PlatformIO initialization file.

Figure 5. PlatformIO initialization file: platformio.ini

Add the following line to the platformio.ini file, as shown in Figure 6:

board_build.bitstream_file = [RVfpgaPath]/RVfpga/src/rvfpganexys.bit

This line indicates where PlatformIO should find the bitstream file to load onto the FPGA. You will use the RVfpgaNexys bitstream distributed with the Getting Started Guide at:  [RVfpgaPath]/RVfpga/src/rvfpganexys.bit. Press Ctrl-s to save the platformio.ini file.

Figure 6. Add location of RVfpgaNexys bitstream file (rvfpganexys.bit)

Remember that a more complete platformio.ini file was used in the examples used in the Getting Started Guide. If you want to use any functionality that requires extra commands (such as the path to the Verilator simulator, the configuration of the serial console, the whisper debug tool, etc.), you can use the platformio.ini from those examples.

Step 2. Write a RISC-V assembly language program

Now you will write a RISC-V assembly program. Click on File → New File (see Figure 7).

Figure 7. Add file to project

A blank window will open. Type (or copy/paste) the following RISC-V assembly program into that window (see Figure 8). This program is also available in:

[RVfpgaPath]/RVfpga/Labs/Lab02/ReadSwitches.S

// memory-mapped I/O addresses

# GPIO_SWs   = 0x80001400

# GPIO_LEDs  = 0x80001404

# GPIO_INOUT = 0x80001408

.globl main

main:

main:

li t0, 0x80001400   # base address of GPIO memory-mapped registers

li t1, 0xFFFF       # set direction of GPIOs

# upper half = switches (inputs)  (=0)

# lower half = outputs (LEDs)     (=1)

sw t1, 8(t0)        # GPIO_INOUT = 0xFFFF

repeat:

lw   t1, 0(t0)      # read switches: t1 = GPIO_SWs

srli t1, t1, 16     # shift val to the right by 16 bits

sw   t1, 4(t0)      # write value to LEDs: GPIO_LEDs = t1

j    repeat         # repeat loop

Figure 8. Enter RISC-V assembly program

The assembly code must contain the following lines at the beginning of the code:

.globl main

main:

The .globl assembler directive makes the label visible in all linked files. The boot code (~/.platformio/packages/framework-wd-riscv-sdk/board/nexys_a7_eh1/startup.S) will configure the system and jump to this label (main). The debugger will set a temporary breakpoint there when it begins.

This RISC-V assembly program is the same example program as in Lab 1, but this time written in RISC-V assembly. It sets the direction of the inputs and outputs of the general-purpose I/O (GPIO) and then repeatedly reads the value of the switches and writes that value to the LEDs.

After entering the program into the pane, press Ctrl-s to save the file. Name it ReadSwitches.S and save it to the src folder of the Project1 directory (see Figure 9).

Figure 9. Save file as ReadSwitches.S

Step 3. Download RVfpgaNexys onto Nexys A7 FPGA board

You will now download RVfpgaNexys onto the Nexys A7 FPGA board. Follow the instructions for downloading RVfpgaNexys as described in the GSG and in Lab 1 – repeated here for convenience.

Download RVfpgaNexys onto the Nexys A7 board by clicking on the PlatformIO icon in the left menu ribbon , then expand Project Tasks → env:swervolf_nexys → Platform and click on Upload Bitstream.

As an alternative you can download RVfpgaNexys using a PlatformIO terminal window by clicking on the PlatformIO: New Terminal button () at the bottom menu of the PlatformIO window, and then typing (or copying) the following into the PlatformIO terminal:

pio run -t program_fpga

Step 4. Compile, download, and run RISC-V assembly program

Now that RVfpgaNexys is running on the board, you will compile your program, download it onto RVfpgaNexys, and run/debug it. If VSCode is not already open, open it. Your last project, Project1, should automatically open. If not, make sure the PlatformIO extension is open and click on File → Open Folder and select (but don’t open) Project1, that you created earlier in this lab.

Click on the Run button in the left menu ribbon and then click on the Start Debugging button (see Figure 10).

Figure 10. Run program on RVfpgaNexys

The program will download onto RVfpgaNexys, which is running on the FPGA on the Nexys A7 board. Now you can begin running and debugging the program (see Figure 11).

Figure 11. Program running on RVfpgaNexys

As described in the RVfpga Getting Started Guide and Lab 1, use the debugging toolbar and Debugger options to run and manage the program. For example, you could set a breakpoint at line 17 (by clicking just to the left of the line number) and then view register t1 as the value of the switches is loaded into it. When you stop the debugging session by pressing the Stop button  (or Shift - F5), the debugging session ends, but the program continues running on RVfpgaNexys.

# Exercises

Now create your own RISC-V assembly programs by completing the same exercises as in Lab 1, but this time in RISC-V assembly instead of C. The exercise descriptions are repeated below for your convenience.

Remember that if you leave the Nexys A7 board connected to your computer and powered on, you do not need to reload RVfpgaNexys onto the board between running different programs. However, if you turn off the Nexys A7 board, you will need to reload RVfpgaNexys onto the board using PlatformIO.

Remember as well that you can run these programs on simulation, using Verilator or Whisper.

Exercise 1. Write a RISC-V assembly program that flashes the value of the switches onto the LEDs. The value should pulse on and off slow enough that a person can view the flashing. Name the program FlashSwitchesToLEDs.S.

Exercise 2. Write a RISC-V assembly program that displays the inverse value of the switches on the LEDs. For example, if the switches are (in binary): 0101010101010101, then the LEDs should display: 1010101010101010; if the switches are: 1111000011110000, then the LEDs should display: 0000111100001111; and so on. Name the program DisplayInverse.S.

Exercise 3. Write a RISC-V assembly program that scrolls increasing numbers of lit LEDs back and forth until all of the LEDs are lit. Then the pattern should repeat. Name the program ScrollLEDs.S.

The program should cause the following to occur:

- First, one lit LED should scroll from right to left.

- Once it reaches the left-most LED, two lit LEDs should scroll from left to right and then right to left.

- Once those two LEDs reach the left-most LED,  three lit LEDs should scroll from left to right then right to left.

- Then four lit LEDs should scroll.

- And so on, until all the LEDs are lit.

- Then the pattern should repeat.

Exercise 4. Write a RISC-V assembly program that displays the unsigned 4-bit sum of the 4 least significant bits of the switches and the 4 most significant bits of the switches. Display the result on the 4 least significant (right-most) bits of the LEDs. Name the program 4bitAdd.S. The fifth bit of the LEDs should light up when unsigned overflow occurs (that is when the carry out is 1).

Exercise 5. Write a RISC-V assembly program that finds the greatest common divisor of two numbers, a and b, according to the Euclidean algorithm. The values a and b should be statically defined variables in the program. Name the program GCD.S. Here is some additional information about the Euclidean algorithm: https://www.khanacademy.org/computing/computer-science/cryptography/modarithmetic/a/the-euclidean-algorithm. You can also simply google “Euclidean algorithm”.

Exercise 6. Write a RISC-V assembly program that computes the first 12 numbers in the Fibonacci sequence, and stores the result in a finite vector (i.e. array), V, of length 12. This infinite sequence of Fibonacci numbers is defined as:

V(0)=0, V(1)=1, V(i)=V(i-1)+V(i-2) (where i=0,1,2...)

In words, the Fibonacci number corresponding to element i is the sum of the two previous Fibonacci numbers in the series. Table 6 shows the Fibonacci numbers for i = 0 to 8.

Table 6. Fibonacci series

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V | 0 | 1 | 1 | 2 | 3 | 5 | 8 | 13 | 21 |

The dimension of the vector, N, must be defined in the program as a constant. Name the program Fibonacci.S.

Exercise 7. Given an N-element vector (i.e., array), A, generate another vector, B, such that B only contains those elements of A that are even numbers greater than 0. For example: suppose N = 12 and A = [0,1,2,7,-8,4,5,12,11,-2,6,3], then B would be: B = [2,4,12,6]. Name the program EvenPositiveNumbers.S.

Exercise 8. Given two N-element vectors (i.e., arrays), A and B, create another vector, C, defined as:

C(i) = |A[i] + B[N-i-1]|, i = 0,..,N-1.

Write a program in RISC-V assembly that computes the new vector. Use 12-element arrays in your program. Name the program AddVectors.S.

Exercise 9. Implement the bubble sort algorithm in RISC-V assembly. This algorithm sorts the components of a vector in ascending order by means of the following procedure:

- Traverse the vector repeatedly until done.

- Interchanging any pair of adjacent components if V(i) > V(i+1).

- The algorithm stops when every pair of consecutive components is in order.

Use 12-element arrays to test your program. Name the program BubbleSort.S.

Exercise 10. Write a program in RISC-V assembly that computes the factorial of a given non-negative number, n, by means of iterative multiplications. While you should test your program for multiple values of n, your final submission should be for n = 7. n should be a variable that is statically defined within the program. Name the program Factorial.S.